Derivation: the creation of requirement based on an analysis of higher-order requirements (ex. availability requirements)

Allocation: the assignment of a requirement to one or more system elements within the architecture. (ex. thrust requirement > propulsion requirements)

Flow Down: assigning requirement to one or more lower-level elements. (ex. water resistant requirement > all elements exposed to outside environment)

Traceability: the creation of linkage from one system elements to others (ex.  mtbf requirements > availability requirements)

System engineer must know how:

- test able

- Produced

- packaged

- transported

Pre-planned product improvement requirements (P3I)

Sys Engg mush work with reliability SMEs

RFA - request for action

RFI - request for information

RVTM - requirements verification and traceability matrix

How to write requirement statement:

- Specific intent

- Desired capability

- Desired level of performance

- Functions or behavioral characteristics

- Not specifying how functions/behaviors are performed (calling for specific components)

- requirement should not specify specific piece of hardware of software into the system

- Determining proper hardware or software is not job description of the systems engineer. It is job of SMEs.

- each requirement should be unambiguous (all parties have same understanding of the requirements)

- requirement should be complete (stand alone, independent of other requirements, avoid cross referencing other requirements, avoid pronoun)

- requirement must be feasible (should not request impossible or unrealistic behavior or feature, some time all req are individually feasible but collection of them are not feasible)

- requirements should be verifiable (analysis, demonstration, test or inspection)

- requirement can use illustration/graphics

Architecture Definition

- it is abstract representation of system elements

- architecture should be implementation free

- is being performed as enabled for other process

- It is primary artifact for the system requirements specifications etc.

Primary goal for architecture definition:

- produce alternate concepts

- compare concepts in trade studies

- select one or more concept alternative(s)

Inputs to architecture definitions:

- Life cycle concepts

- system function definition

- system requirements

- system functional interface identification

- System requirements traceability

- Updated RVTM

- Design traceability

- Interface definition update identification

- Life cycle constraints.

Goes into architecture definition

- Prepare for architecture definition

- Develop architecture viewpoints

- Develop models and views

- Relate the architecture to the design

- Assess architecture candidates

- Manage the selected architecture

Output of architecture definition

- Architecture definition strategy

- System architecture description

- System architecture rationale

- Documentation tree

- Preliminary interface definition

- Technical performance measure needs and data

- Architecture traceability

- Architecture definition record.

Architecture views

- operational (user's perspective)

- logical (acquirer/customer's perspective)

- Physical (designer's perspective)

Architecting is abstract/inductive. More art than science designing is deductive. More science than art

Architecture is not a modeling language.

Model based systems engineering (MBSE)

- systems engineering (requirement development, functional and mission development, physical definition)

- MBSE tool

- MBSE language (sysML)

A thing that performs a function is called physical including software

System model domains -

- Requirement domain: description of mandatory things your widget needs to do

- Behavioral domain: all the ways your widget should behave goes

- Structural domain: what your widget is made out of is in structural domain

Interfaces: (forces, energy, data, signals or materials)

SysML - Systems modeling language and UML - unified modeling language:

- Modeling language: artificial language of communicating concept in standardized way

- agreed upon grammar and vocabulary

- mostly graphical

Unified modeling language:

- general purpose modeling language for software engineer that is intended to provide a standard way to visualize the design of a software system

System modeling language:

- general purpose modeling language for systems engineer that is intended to provide a standard way to visualize the design of a system (hardware and software)

- it adds two new diagram (requirements and parametric)

Type of diagrams:

- Structure Diagram types

  - Bdd - block definition diagram

  - ibd - internal body diagram

  - par - parametric diagram

  - pkg - package diagram

- Behavior diagram types

  - act - activity diagram

  - seq - sequence diagram

  - stm - state machine diagram

  - ucd - use case diagram

- Requirement diagram type

  - req - requirement diagram

BDD (block definition diagram)

- it is a structure of system showing system blocks, their attributes, property and interactions

- analogue to class diagram in UML

- good for systems hierarchy

IBD (internal block diagram)

- internal structure of single structure of the block

- inputs on the perimeter of diagram are brought in

Activity diagram

- Depicts system behaviors, sequencing, decision and other logic between system behaviors

- functional interfaces (force, energy, data, signal, material)

- control flow is dotted line and object flow is solid line

- decision node needs logic

Sequence diagram (never saw using this diagram)

- blocks

- relation between blocks and actors

- synchronous messages or call messages

State machine

- behavioral diagram

- on state, off state, degraded state

- nested state (substate)

- start point and end point are important

Parametric diagram

- performance, reliability and SWaP analysis (size weight and power)

Package diagram

- hierarchy diagram

- architecture diagram

- requirement domain, behavior domain and structural domain\

UML diagrams:

- Behavioral Diagram

- Activity diagram

- State machine diagram

- Interaction diagram

- Timing diagram

- Interaction overview diagram

- Communication diagram

- Sequence diagram

- Use case diagram

Structure diagram

- component diagram

- class

- composite

- deployment

- package

- object

Behavioral/functional domain:

- function/behavior and activity are synonymous in sysengg

Function is an action performed by system element that converts inputs to output. Function:

- uses input

- generates output

- based on requirements/context/scenario

- decomposes/decomposed by sub-function

- performs/performed by component

Activities vs action

- Activities are overarching system behavior, composed of actions. These are decomposed into lower-level actions.

- Actions are leaf level system behavior. Actions are not decomposed. Actions are basic units of functionality.

Activity must have at-least one input and one output.

Outputs are always different than inputs

Sequence diagram always draws relation between physical elements from functional perspective.

Functional block diagram:

- it depicts relation between internal elements to external entity

Use case:

it is a list of steps user will most likely deploy, operate, support or dispose of a system to achieve a desired goal.

- helps refine requirements.

- foster top-down design

- preempt errors and faults

- save time and money preventing faults/ errors from existing

- help decompose the problem into more manageable segment

Elements of an usecase:

- Actors: a stakeholder / one or more actors.

- Summary: description of an use case in paragraph from

- goal: primary outcome desired by an actor

- pre-condition: things that must be true prior to starting the use-case guarantees: the least the system can do for the actor, if the goal is not met.

- trigger: step 1 in course of an environment

- course of event: numbered list of actions the actor/s and system take to accomplish the goal.

- alternative path:

Primary source of function:

- Requirement specifications

- Context Diagram

- Life Cycle Concepts

- Legacy and predecessor system functions

- Parent functions

Top-down systems engineering:

- The behavioral domain is bases on

  - the requirements domain

  - the mission domain

  - itself

- the behavioral domain is not based on

  - the physical domain

Top-down sys engg

- Needs, technology driver, context basis of:

  - life cycle concepts and requirements basis of:

    - functions basis of:

      - hardware/ software

Hardware and software are based on functions. Functions are not based on hardware and software.

Stakeholder needs and requirements definition focus:

- high-risk systems ands subsystem entities

- defining systems and subsystem performance requirements/MOEs

- defining the lifecycle concepts

Architecture definition focus:

- Decompose system and subsystem functions until makes/buy decision can be made at component level

- Mitigate function-related risks into the green

- Complete the rest of the functional architecture for non-primary-mission scenarios, fail modes and recovery operations

- Heavy use of analysis, modeling and simulations

Interfaces:

- Force

- Energy

- Data

- Signal

- Material

Functional block diagram does not show logic or sequence, but it shows activities from one to other.

Primary source of function:

- Requirement specifications

- Context diagram

- Life cycle concepts

- Legacy and predecessor system functions

- Parent function

Requirement development -> functional and mission development -> Physical design

Validating system concept:

- Evaluate system performance using models, sims and prototypes

- Perform system level conceptual analysis and simulation “effectiveness” runs through missions, scenarios and use cases

- Primary focus is on mission critical performance attributes (KPP)

- Prototype may be used to demonstrate critical technologies

- Rely on independent evaluations and user feedback!

System/ product development process (INCOSE)

- Agreement Process

  - Acquisition process

  - Supply process

- Enterprise Process:

  - Enterprise environmental management process

  - Investment management process

  - System lifecycle management process

  - Resource management process

  - Quality management process

- Technical management process:

  - Project planning process

  - Project assessment and control process

  - Decision management process

  - Risk management process

  - Configuration management process

  - Information management process

  - Measurement process

  - Quality assurance process

- Technical process:

  - Business/ mission analysis process

  - Stakeholders needs and requirement definition process

  - System requirement definition process

  - Architecture definition process

  - System analysis process

  - Design definition process

  - Implementation process

  - Integration process

  - Verification process

  - Transition process

  - Validation process

  - Operation process

  - Maintenance process

  - Disposal process

- Organizational Project-enabling processes:

  - Life cycle model management process

  - Infrastructure management process

  - Portfolio management process

  - Human resource management process

  - Quality management process

  - Knowledge management process

# Design Definitions

Inputs:

- Lifecycle concepts

- System function definitions

- System requirements

- Functional interface identification

- System architecture description

- System architecture rationale

- Preliminary interface definition

- Preliminary TPM needs and data

- Architecture traceability

- Interface definition update identification

- Implementation traceability

- Life cycle constraints

Process:

- Prepare for design definition

- Establish design characteristics and design enablers related to each system elements

- Assess alternatives for obtaining system elements

- Manage the design

Output:

- Design definition strategy

- System design description

- System design rationale

- Interface definition

- TPM needs and data

- Design traceability

- System element description

- Design Definition record

# System Analysis Process

Concept

- Business/ mission analysis

- Stakeholder needs/ requirement definitions

- System requirement definition

  - These are reviewed and approved at system requirement gate review

- Architecture definition

Agreement process: In this process company hire potential external supplier to help with the system requirement generation and analysis may be.

System Development:

- Design definition

- Implementation

- Integration

- Verification

System Production:

- Validation

- Transition

Utilization

- Support

- Operation

Maintenance:

Request for Proposal:

- Also called as solicitation

- Formal means of initiating a contract for products and services from experts

- Can include purchasing already-existing systems, as-is

- Can include only labor

- Can include developing and end items

- Can span relatively brief periods of item, or

- Can span quite long periods of time.

Statement of work: services and service-related product

System requirement specification: end item description

Technology readiness level: it is a measurement scale of how mature (ready) as system segment is

- Applicable to individual system elements

- Also applicable to the system as whole

TRL1 : is least mature system

TRL9 : is most mature system

TRL1:

- An idea, based on known scientific research

- Expressed in analytical terms

- Documented in scientific white papers

TRL2:

- Rudimentary analytical studies

- Technology unproven/ does not exist

- Relies on analysis of scientific principle

- Idea validated with stakeholders

  - May invest in proof-of-concept development

TRL3:

- Proof of concept development

- Use of analysis and lab experiments

- Software logic, pseudo-code (UML model)

TRL4:

- Lab testing

- Validated concept against original idea

  - Without rigorous testing

  - Just to see if it works/ is feasible

- May be integrated with other elements

TRL5:

- Mature the design

  - Design-in the “ilities”

- Software alpha release

  - Focus on functionality “does it work?”

TRL6:

- Mature the prototype

- Entity exhibits full functionality of primary characteristics

- Software beta release

  - Focus on usability/ use satisfaction

- Sunny day scenario tests

TRL7:

- Demonstrated in controlled operational scenarios (user group sessions)

- System possesses primary features, functions, characteristics

- More rigorous environmental/ rainy day testing

TRL8:

- Baseline the design

- Build test articles, conduct operational testing

TRL9:

- Finalized product

- Fielded, in use by actual users

- Entity performs mission in real environment with real user(s)

Systems Engineer needs to consider forward integration into the production line and also to retrofit

Lead systems engineer/ chief engineer/ technical director

SEMP: Systems Engineering Management Plan

Interfaces:

- Human-machine interfaces

- Environmental interfaces

- Segment interfaces

- Software module interfaces

Interfaces are showed in the segment level context diagram.

Behavioral diagram:

- Activity diagram, sequence diagrams and state machine diagram are the three options that sysml offers to specify the system behavior.

- The behavior diagram is intended to provide clarity for example about the internal process, business process, or the interaction of different systems.

Loosely bound and tightly coupled functions: BAD example

Tightly bound and loosely coupled functions: GOOD example

Test types in test plan:

- Segment level (unit)

- System integration

- Stress/ failure mode

- Validation (user) tests

- Regression (re-test)

- System-level (post-integration)

Unit test: it is a software development process to test smallest testable part of an application called units, are individually scrutinized for proper operation.

Integration tests: white-box testing and black-box testing

Regression test:

- it is a type of software testing conducted after a code update to ensure that the update introduced no new bugs.

- It is re-running functional and non-functional tests to ensure that previously developed and tested software still performs as expected after a change. If not, that would be called a regression.